Forestry BMP Verification –Principals and Sample Protocols (in subtext)

Discussion Draft for Forestry Workgroup 3/6/2012

Why Verification:  Given the ever increasing importance that accounting for BMPs is taking on within the partnership—Bay TMDL reasonable assurance, two-year milestones, offsets, tradable credits, etc. —Chesapeake Bay partners need to have assurance that the practices reported are sound, or verifiable.

Verification Framework:

- Partnership agreement on a set of verification principles to guide the jurisdictions’ development of verification programs

- Partnership agreement on overall set of source sector-specific verification protocols. These protocols would be developed by the sector workgroups (e.g., Forestry Workgroup), approved by the Partnership, and used (tailored as needed) by the jurisdictions.

- Establishment of a Verification Panel charged by the Partnership to review and make recommendations back to the Partnership

Note: Jurisdictions will draw up their own Verification Program based on this framework.

Urban RFB and Tree Planting BMP Principles

- Ensure tree planting acreage represents a net gain in overall tree cover

  - Jurisdictions with an urban forestry partner/staff trusted by state forestry agency—who also likely:  have a UTC assessment, goal, and implementation plan, plant 1-2” caliper trees to improve chances of survival, have ordinances that protect urban trees, and conduct survival/health monitoring of plantings— could receive full credit (100%) for this practice as reported.  Baseline survey and an established program are both needed to assure a net gain.

    - Jurisdictions without a trusted urban forestry partner/staff reporting could be discounted 30%?? for uncertainty of survival/net gain in tree cover

    - An additional discount (60% ??) is applied if a proxy for trees  “trees sold” or simply a website submission is used and trees were not viewed by a professional

  - Maintain detailed record-keeping on tree planting at local level, but simplify and standardize information for reporting up to state-> NEIEN

  - Avoid double-counting and avoid counting tree planting for mitigation

    - Does tree planting under MD’s FSA warrant an exception?

    - Tree plantings that do not survive will be replaced within a year or reported as such through the localstate record keeping.

    - Something to keep in mind-- urban tree planting is still largely undercounted.  With the verification process is an opportunity to educate about tree planting and improve tracking and reporting from local governments/watershed groups up through state, as these verification principles are implemented.

  - States will verify the urban tree planting practice by sampling 10% of reporting jurisdictions after 2-5 years, using high-resolution imagery tools or an equivalent measure.  These will generally be in the 1a category above, or jurisdictions reporting substantial tree planting.

Agricultural Riparian Forest Buffer Tree Planting BMP Principles

- Avoid double-counting

  - State forestry agency reviews cost-share project data from USDA/USGS prior to NEIEN input

  - Work to establish a unique identifier for each project to avoid duplicate records

- Report length and/or width of project (not just acres)- this is important to more accurately capture the WQ effectiveness.  Narrower buffers (>35’ and <100’) will eventually be discounted)

--Could NRCS/FSA begin tracking buffer length or width not just acres?

- Ensure projects are properly installed and maintained

--10% of new, non-cost share RFB installations should be monitored (in keeping with USDA cost-share practice)

- Remove practice from database/model if it is no longer there

--Sample forest buffer at 15 yo to verify establishment

- Need to differentiate re-enrolled CREP acres

- Practice needs to represent a net gain in acres to count

  - RFB baseline for a state should be ascertained using high resolution imagery and the Land Image Analyst or other tool.  Re-sample every 5-6 years to verify net gain.

Forest Harvesting BMP Principle

- Forest harvest BMP rate of implementation needs to be determined every 5 years by state (e.g., sampling or survey)

  - Could have field checklist of how practice was implemented, verified

  - Focus on forest harvesting BMPs that are most important to water quality?

  - States that don’t know rate of private land harvest should be discounted…

  - If states have information on forest harvesting, they can submit acres and override the 1% harvest rate assumption

  - States with regulations and monitoring programs in place could use that info in lieu of forest harvest BMP implementation sampling

  - Adaptive management –BMP monitoring to determine if a practice worked (should this be incorporated into verification?)