### The Control File

### __________________________

The control file contains the parameters used to run CE-QUAL-ICM.  It consists of card images 80 characters in length.  The format of the input files has been developed in order to take advantage of a full-screen text editor.  The control file begins with six lines, which can be used for file identification and are ignored by the program.  The rest of the file consists of a line, which contains the card identification and the names of the FORTRAN variables associated with the input card.  The FORTRAN names are right-justified according to the field widths associated with the input variable.  The next line consists of the actual input values.  There are 10 input fields associated with each card although the first field is not used.  Each field has a length of eight characters.  The next line is left blank for ease of display in a full-screen editor.

There are no optional cards in the control file - each card is required although there may be no values associated with the card.  The following pages contain a description of each card.  All character inputs must be capitalized except the TITLE cards or the variable will take on the default value.

Title Cards (TITLE)

Field		Name		Value						Description

1-72		TITLE		Character				Text for identification

of simulation

There are six title cards that help identify the model run.  Each line contains up to 72 characters of text.  The title cards appear in every output file except for restarts.  Uses for the title cards include identifying the application, the dates of the application, the date of the simulation, and any other information specific to the simulation.

# EXAMPLE

TITLE C ......................................Title.....................................

Start revisions of Lake Washington model

Fix-ups first, then major revisions

Run from /disk3/cerco/lake_wash

February 25, 2001

Mass balance and Sensitivity runs of 30-box 3-D grid.

Initial code from /cc1/cerco/SENS43_new_grid (Ches Bay)

Geometry Definition (GEOM DEFINE)

Field		Name		Value						Description

1			NB			Integer				Number of boxes in grid

2			NSB		Integer				Number of surface boxes in grid

3			NQF		Integer				Number of flow faces in grid

4			NHQF		Integer				Number of horizontal flow faces  												in grid

5			NSHQF		Integer				Number of horizontal flow faces  												in surface layer of grid

6			NL			Integer				Maximum number of layers in

grid

This card specifies the dimensions of the water-quality model grid.  The user must take care that the values specified here are less than or equal to the corresponding PARAMETER statements in the file WQM_COM.INC.

EXAMPLE

GEOM DEFINE   NB     NSB     NQF    NHQF   NSHQF      NL

12177     655   32869   21347    1177      41

Time Control (TIME CON)

Field		Name		Value						Description

1			TMSTRT	Real					Starting simulation date

(Julian day)

2			TMEND	Real					Ending simulation date

(Julian day)

This card specifies the starting and ending dates of the simulation.

# EXAMPLE

TIME CON  TMSTRT   TMEND

0.0  1095.5

Number of Timestep Intervals (# DLT)

Field		Name		Value						Description

1			NDLT		Integer					Number of timestep

intervals

The model provides the option to vary the time step.  The time step may be varied through the autostepping option or at discrete, user-specified intervals.  This card specifies the number of intervals in which the timestep, maximum timestep, and/or the fraction of the calculated timestep vary.

This card group establishes a pattern followed by successive groups which govern output.  The first card establishes that time steps are assigned once.  The second card names the time as day 0.  The third card assigns a time step of 2400 seconds at day 0.

EXAMPLE

# DLT       NDLT

1

Timestep Date (DLT DAY)

Field		Name		Value						Description

1			DLTDAY	Real					Date of timestep interval 													(Julian day)

This card specifies the intervals in which the timestep, maximum timestep, and/or fraction of the maximum calculated timestep may vary.

EXAMPLE

DLT DAY     DLTD    DLTD    DLTD    DLTD    DLTD    DLTD    DLTD    DLTD    DLTD

0.0

Timestep Value (DLT VAL)

Field		Name		Value						Description

1			DLTVAL	Real							Timestep (sec)

This card specifies the initial timestep if autostepping is used or the values for the timestep if autostepping is turned off.

EXAMPLE

DLT VAL   DLTVAL  DLTVAL  DLTVAL  DLTVAL  DLTVAL  DLTVAL  DLTVAL  DLTVAL  DLTVAL

2400.0

Maximum Timestep (DLT MAX)

Field		Name		Value						Description

1			DLTMAX	Real						Maximum timestep (sec)

This card specifies the maximum value for the timestep if autostepping is turned on.

EXAMPLE

DLT MAX   DLTMAX  DLTMAX  DLTMAX  DLTMAX  DLTMAX  DLTMAX  DLTMAX  DLTMAX  DLTMAX

2400.0

Timestep Fraction (DLT FTN)

Field		Name		Value						Description

1			DLTFTN	Real				Fraction of calculated timestep used

This card specifies the fraction of the timestep used if autostepping is turned on.  The autostepping algorithm estimates the maximum timestep in order to maintain stability.  Since this is an estimate, DLTFN can be used to decrease the timestep if the model becomes unstable when autostepping is turned on.

EXAMPLE

DLT FTN   DLTFTN  DLTFTN  DLTFTN  DLTFTN  DLTFTN  DLTFTN  DLTFTN  DLTFTN  DLTFTN

0.90

Hydrodynamic Model Timestep (HM DLT)

Field		Name		Value						Description

1			AHMDLT	Real			Hydrodynamic update interval (sec)

2			FILGTH	Real			Ending date of hydrodynamic file

(Julian day)

This card specifies the interval at which hydrodynamic information is input to the model.  This information is employed only if "BINARY" or "DEPTH-AVERAGE" options are specified (see card HYD MODEL).  The present formulation requires a constant update interval.  The ending date of the hydrodynamic input (HMEND) is necessary to synchronize elapsed time in the model, time determined from hydrodynamic updates, and update times specified on various input files.  If multiple hydrodynamic files are employed, they should be as close to one year in length as possible.

EXAMPLE

HM DLT    AHMDLT   HMEND

3600.0   365.0

Snapshot Output Control (SNAPSHOT)

Field		Name		Value						Description

1			SNPC		Character			Specifies if output is written to 												snapshot file

2			NSNP		Integer				Number of snapshot intervals

This card specifies if information is output to the snapshot file.  The output interval is variable.  The card also lists the number of intervals in which the frequency varies.

The first card in this card group indicates that snapshot frequency is specified one time.  The second card indicates frequency is specified on day 0.  The final card indicates snapshots are put out at a frequency of every 365 days starting at day 0.

EXAMPLE

SNAPSHOT    SNPC    NSNP

ON       1

Snapshot Date (SNAP DAY)

Field		Name		Value						Description

1-9			SNPD		Real					Date of snapshot interval 													(Julian day)

This card specifies the intervals at which the snapshot output varies.  If there are more intervals than can be specified on one line, then they are continued on the next line.

EXAMPLE

SNAP DAY    SNPD    SNPD    SNPD    SNPD    SNPD    SNPD    SNPD    SNPD    SNPD

0.0

Snapshot Frequency (SNAP FREQ)

Field		Name		Value						Description

1-9			SNPF		Real					Snapshot output frequency 												(Julian day)

This card specifies the frequency at which snapshot information is output.  If there are more values than can be specified on one line, then they are continued on the next line.

EXAMPLE

SNAP FRQ    SNPF    SNPF    SNPF    SNPF    SNPF    SNPF    SNPF    SNPF    SNPF

365.0

Plot Output Control (PLOT)

Field		Name		Value						Description

1			PLTC		Character			Specifies if output is written to

plot file

2			QPLTC		Character			Specifies if water quality

information is output

3			SPLTC		Character			Specifies if sediment information

is output

4			SAVPLTC	Character			Specifies if Submerged Aquatic

Vegetation information is output

5			NPLT		Integer				Number of plot intervals

This card specifies if information is output to the plot file and the number of intervals in which the frequency of the output varies.

# EXAMPLE

PLOT        PLTC   QPLTC   SPLTC SAVPLTC    NPLT

OFF      ON      ON     OFF       1

Plot Date (PLOT DAY)

Field		Name		Value				Description

1-9			PLTD		Real				Date of plot interval

(Julian day)

This card specifies the intervals at which the plot output varies.  If there are more intervals than can be specified on one line, then they are continued on the next line.

EXAMPLE

## PLOT DAY    PLTD    PLTD    PLTD    PLTD    PLTD    PLTD    PLTD    PLTD    PLTD

0.0

Plot Frequency (PLOT FREQ)

Field		Name		Value				Description

1-9			PLTF		Real				Plot output frequency

(Julian day)

This card specifies the frequency at which plot information is output.  If there are more values than can be specified on one line, then they are continued on the next line.

EXAMPLE

PLOT FREQ    PLTF    PLTF    PLTF    PLTF    PLTF    PLTF    PLTF    PLTF    PLTF

10.0

Average Plot Output Control (AV PLOT)

# Field		Name		Value				Description

1			APLC		Character		Specifies if output is written to

average plot file

2			NAPL		Integer			Number of average plot intervals

This card specifies if information is output to the average plot file and the number of times at which averaging interval varies.

The first card in this card group indicates that averaging interval is specified once.  The second card indicates the interval is specified on day 0.  The final card indicates the averaging interval is 1.0 day starting at day 0.

EXAMPLE

AV PLOT     APLC    NAPL

ON       1

Average Plot Date (AVPLT DAY)

Field		Name		Value			Description

1-9			APLD		Real			Date of average plot interval

(Julian day)

This card specifies the intervals at which the average plot output varies.  If there are more intervals than can be specified on one line, then they are continued on the next line.

# EXAMPLE

AVPLT DAY   APLD    APLD    APLD    APLD    APLD    APLD    APLD    APLD    APLD

0.0

Average Plot Frequency (AVPLT FREQ)

Field		Name		Value						Description

1-9			APLF		Real			Average plot output frequency

(Julian day)

This card specifies the frequency at which average plot information is output.  If there are more values than can be specified on one line, then they are continued on the next line.

EXAMPLE

AVPLT FREQ   APLF    APLF    APLF    APLF    APLF    APLF    APLF    APLF    APLF

1.0

Transport Flux Output Control (TRAN FLUX)

Field		Name		Value						Description

1			TFLC		Character		Specifies if output is written

to transport flux file

2			NTFL		Integer			Number of transport flux intervals

This card specifies if information is output to the transport flux file and the number of intervals in which the frequency of the output varies.  Transport flux is averaged similar to the Average Plot file.

EXAMPLE

TRAN FLUX   TFLC    NTFL

OFF       1

Transport Flux Date (FLUX DAY)

Field		Name		Value			Description

1-9			TFLD		Real			Date of transport flux interval

(Julian day)

This card specifies the intervals at which the transport flux output varies.  If there are more intervals than can be specified on one line, then they are continued on the next line.

# EXAMPLE

FLUX DAY    TFLD    TFLD    TFLD    TFLD    TFLD    TFLD    TFLD    TFLD    TFLD

0.0

Transport Flux Frequency (FLUX FREQ)

Field		Name		Value			Description

1-9			TFLF		Real			Transport flux output frequency

(Julian day)

This card specifies the frequency at which transport flux information is output.  If there are more values than can be specified on one line, then they are continued on the next line.

# EXAMPLE

FLUX FREQ   TFLF    TFLF    TFLF    TFLF    TFLF    TFLF    TFLF    TFLF    TFLF

91.25

Kinetics Flux Output Control (KIN FLUX)

Field		Name		Value						Description

1			KFLC		Character		Specifies if output is written

to kinetics flux file

2			NKFL		Integer			Number of kinetics flux intervals

This card specifies if information is output to the kinetics flux file and the number of intervals in which the frequency of the output varies.  Kinetics fluxes are averaged similar to the Average Plot file

EXAMPLE

KIN FLUX    KFLC    NKFL

OFF       1

Kinetic Flux Date (FLUX DAY)

Field		Name		Value			Description

1-9			KFLD		Real			Date of kinetic flux interval

(Julian day)

This card specifies the intervals at which the kinetic flux output varies.  If there are more intervals than can be specified on one line, then they are continued on the next line.

EXAMPLE

FLUX DAY    KFLD    KFLD    KFLD    KFLD    KFLD    KFLD    KFLD    KFLD    KFLD

0.0

Kinetic Flux Frequency (FLUX FREQ)

Field		Name		Value			Description

1-9			KFLF		Real			Kinetic flux output frequency

(Julian day)

This card specifies the frequency at which kinetic flux information is output.  If there are more values than can be specified on one line, then they are continued on the next line.

EXAMPLE

FLUX FREQ   KFLF    KFLF    KFLF    KFLF    KFLF    KFLF    KFLF    KFLF    KFLF

30.42

Oxygen Plot Output Control (OXY PLOT)

Field		Name		Value				Description

1			OPLC		Character		Specifies if output is written

to oxygen plot file

2			NOPL		Integer			Number of oxygen plot intervals

3			NOINT		Integer			Number of oxygen concentration

intervals

This card specifies if information is output to the oxygen plot file and the number of intervals in which the frequency of the output varies.  In addition, it specifies the oxygen concentration ranges used in determining oxygen volume-days.  Oxygen Plot output is averaged in a manner similar to the Average Plot file.

EXAMPLE

OXY PLOT    OPLC    NOPL   NOINT

OFF      42       5

Oxygen Intervals (OXY INT)

Field		Name		Value			Description

1-9			OINT		Real			Interval value for determining

oxygen volume-days

This card specifies the concentration ranges used in determining oxygen volume-days.  If there are more values than can be specified on one line, then they are continued on the next line.

In the example below with NOINT=5, the intervals would be defined as

Interval 1	-10.0 <= Dissolved Oxygen < 1.0

Interval 2	1.0 <= Dissolved Oxygen < 3.0

Interval 3	3.0 <= Dissolved Oxygen < 4.0

Interval 4	4.0 <= Dissolved Oxygen < 5.0

Interval 5	5.0 <= Dissolved Oxygen

EXAMPLE

OXY INT     OINT    OINT    OINT    OINT    OINT    OINT    OINT    OINT    OINT

-10.0     1.0     3.0     4.0     5.0

Oxygen Plot Date (OXY DAY)

Field		Name		Value			Description

1-9			OPLD		Real			Date of oxygen plot interval

(Julian day)

This card specifies the intervals at which the oxygen plot output varies.  If there are more intervals than can be specified on one line, then they are continued on the next line.

EXAMPLE

OXY DAY     OPLD    OPLD    OPLD    OPLD    OPLD    OPLD    OPLD    OPLD    OPLD

0.0    50.0   141.0   233.0   324.0   415.0   507.0   598.0   689.0

780.0   872.0   963.0  1054.0  1145.0  1237.0  1328.0  1419.0  1510.0

1602.0  1693.0  1784.0  1875.0  1967.0  2058.0  2149.0  2240.0  2332.0

2423.0  2514.0  2605.0  2697.0  2788.0  2879.0  2971.0  3062.0  3153.0

3244.0  3336.0  3427.0  3518.0  3609.0  3650.0

Oxygen Plot Frequency (OXY FREQ)

Field		Name		Value			Description

1-9			OPLF		Real			Oxygen plot output frequency

(Julian day)

This card specifies the frequency at which oxygen plot information is output.  If there are more values than can be specified on one line, then they are continued on the next line.

In the example below, the 365.0 day frequency is greater than each of the differences in the OPLD values defined in the previous cards.  Thus, the simulation would use the next OLPD (and its associated OPLF) before the current OLPF would be reached.

EXAMPLE

OXY FREQ    OPLF    OPLF    OPLF    OPLF    OPLF    OPLF    OPLF    OPLF    OPLF

365.0   365.0   365.0   365.0   365.0   365.0   365.0   365.0   365.0

365.0   365.0   365.0   365.0   365.0   365.0   365.0   365.0   365.0

365.0   365.0   365.0   365.0   365.0   365.0   365.0   365.0   365.0

365.0   365.0   365.0   365.0   365.0   365.0   365.0   365.0   365.0

365.0   365.0   365.0   365.0   365.0   365.0

Mass Balance Output Control (MASS BAL)

Field		Name		Value				Description

1			MBLC		Character		Specifies if output is written

to mass balance file

2			NMBL		Integer			Number of mass balance intervals

This card specifies if mass balances are computed and the number of intervals in which the frequency of the output varies.  Mass balances are averaged in a manner similar to the Average Plot file.

# EXAMPLE

## MASS BAL    MBLC    NMBL

OFF       1

Mass Balance Date (MBL DAY)

Field		Name		Value						Description

1-9			MBLD		Real			Date of mass balance interval

(Julian day)

This card specifies the intervals at which the mass balance output varies.  If there are more intervals than can be specified on one line, then they are continued on the next line.

# EXAMPLE

## MBL DAY     MBLD    MBLD    MBLD    MBLD    MBLD    MBLD    MBLD    MBLD    MBLD

0.0

Mass Balance Frequency (MBL FREQ)

Field		Name		Value						Description

1-9			MBLF		Real			Mass balance output frequency

(Julian day)

This card specifies the frequency at which mass balance information is output.  If there are more values than can be specified on one line, then they are continued on the next line.

EXAMPLE

## MBL FREQ    MBLF    MBLF    MBLF    MBLF    MBLF    MBLF    MBLF    MBLF    MBLF

30.4

Diagnostics Output Control (DIAGNSTCS)

Field		Name		Value						Description

1			DIAC		Character		Specifies if output is written

to diagnostic file

2			NDIA		Integer			Number of diagnostic intervals

This card specifies if information is output to the diagnostics file and the number of intervals in which the frequency of the output varies.

EXAMPLE

## DIAGNSTCS   DIAC    NDIA

ON       1

Diagnostics Date (DIA DAY)

Field		Name		Value			Description

1-9			DIAD		Real			Date of diagnostics interval

(Julian day)

This card specifies the intervals at which the diagnostics output varies.  If there are more intervals than can be specified on one line, then they are continued on the next line.

EXAMPLE

## DIA DAY     DIAD    DIAD    DIAD    DIAD    DIAD    DIAD    DIAD    DIAD    DIAD

0.

Diagnostic Frequency (DIA FREQ)

Field		Name		Value			Description

1-9			DIAF		Real			Diagnostics output frequency

(Julian day)

This card specifies the frequency at which diagnostics information is output.  If there are more values than can be specified on one line, then they are continued on the next line.

EXAMPLE

## DIA FREQ    DIAF    DIAF    DIAF    DIAF    DIAF    DIAF    DIAF    DIAF    DIAF

1.00

Restart Output (RESTART)

Field		Name		Value						Description

1			RSOC		Character		Specifies if output is written

to restart 	file

2			NRSO		Integer			Number of restart output dates

3			RSIC		Character		Specifies if present simulation is

generated from a restart file

NOTE:  RSIC is read, thus its inclusion is necessary in the Control File, but is a relic variable and is not used in this application.

EXAMPLE

RESTART     RSOC    NRSO    RSIC

## OFF       1     OFF

Restart Date (RST DAY)

Field		Name		Value			Description

1-9			RSOD		Real			Date of restart output

(Julian day)

This card specifies the days at which restart information is output.  If there are more restarts than can be specified on one line, then they are continued on the next line.

EXAMPLE

## RST DAY     RSOD    RSOD    RSOD    RSOD    RSOD    RSOD    RSOD    RSOD    RSOD

364.0

Hydrodynamic Model (HYD MODEL)

Field		Name		Value			Description

1			HYDC		Character		Specifies the type of

hydrodynamic model input

2			PREVAPC	Character		Specifies if precipitation and

evaporation are considered

This card specifies the nature of hydrodynamic inputs to the water quality model.  There are three options for HYDC: "BINARY", "ASCII", and "DEPTH_AV".  The BINARY option is usually specified for large files generated by a hydrodynamic model, e.g. CH3D-WES.  The ASCII option is usually specified for smaller files which are created independent of a hydrodynamic model.  The DEPTH_AV option indicates a two-dimensional (longitudinal-lateral) implementation of the water-quality model with binary input format. PREVAPC specifies if the hydrodynamic model simulated precipitation and evaporation, in which case, hydrodynamic model surface volumes will be used in the water quality model. Otherwise the surface volumes will be calculated internally in the water quality model.

EXAMPLE

HYD MODEL   HYDC PREVAPC

BINARY      ON

Hydrodynamic Solution (HYD SOLTN)

Field		Name		Value						Description

1			SLC		Character		Specifies the type of transport solution

2			CONSC		Character		Specifies the type of conservation     `											used

3			TH			Real				Crank-Nicholson weighting factor

4			MINSTEP	Real				Specifies Minimum Timestep

(seconds)

This card allows the user to specify the type of longitudinal transport scheme that the water quality model uses, how mass is conserved, and the amount of implicit/explicit weighting used in vertical transport. The options for the longitudinal transport scheme are "UPWIND" and "QUICKEST". The upwind option is numerically diffusive which can cause problems during calibration.  The QUICKEST option reduces numerical diffusion but can cause overshoots and undershoots in regions of sharp gradients. The model has the capability to use different hydrodynamic files over the period of a simulation. If the hydrodynamic files are disjointed (not continuous in time), the total volume of the system can change when a new hydrodynamic file is used. The abrupt change in volume may cause mass-balance errors in the water-quality model.  The user can specify if either mass ("MASS") or concentration ("CONC") is conserved when hydrodynamic files are spanned. If “MASS” is chosen the model will conserve mass but may present concentration discontinuity. If “CONC” is chosen the model will maintain concentrations but may not conserve mass. Theta ("TH") specifies the amount of explicit /implicit time weighting in the vertical transport scheme.  A value of zero is fully explicit and a value of one is fully implicit.

# EXAMPLE

HYD SOLTN    SLC   CONSC      TH MINSTEP

QUICKEST    MASS    0.75     5.0

Miscellaneous Controls (CONTROLS)

Field		Name		Value						Description

1			SEDC		Character		Turns on/off predictive sediment 											submodel

2			AUTOC	Character		Turns on/off autostepping

3			VBC		Character		Turns on/off volume balance 												calculations

4			BFOC		Character		Turns on/off output of benthic model 											parameters and fluxes to designated 											output file

5			STLC		Character		Turns on/off particle settling

6			ICIC		Character		Specifies format of initial conditions

7			ICOC		Character		Specifies if initial conditions

are created for use in a subsequent

model run

8			SAVMC	Character		Turns on/off predictive Submerged

Aquatic Vegetation submodel

The first five fields must be specified as " ON" or "OFF".  The sixth field may be "UNIFORM", "VARIED", OR "BINARY".  Descriptions of these options are provided in the description of the Initial Conditions File.  The last two fields must be specified as " ON", or "OFF".

EXAMPLE

CONTROLS    SEDC   AUTOC     VBC    BFOC    STLC    ICIC    ICOC   SAVMC

ON      ON      ON     OFF      ON  BINARY      ON     OFF

Benthos Submodel Controls (CONTROLS)

Field		Name		Value						Description

1			SUSFDC	Character		Turns on/off suspension feeders

2			DEPFDC	Character		Turns on/off deposit feeders

3			LOXC		Character		Turns on/off logistic mortality

function in benthos submodel

These three fields must be specified as " ON" or "OFF".  If SUSFDC and DEPFDC are both “OFF” the value of LOXC is inconsequential since the benthos are both “OFF”.

EXAMPLE

CONTROLS  SUSFDC  DEPFDC    LOXC

OFF     OFF      ON

Dead Sea Case (DEAD SEA)

Field		Name		Value						Description

1			FLC		Character			Turns on/off hydrodynamic flows

2			XYDFC		Character			Turns on/off horizontal diffusion

3			ZDFC		Character			Turns on/off vertical diffusion

This card allows the user to turn off any or all transport processes in the model.  All fields must be specified as " ON" or "OFF".

EXAMPLE

## DEAD SEA     FLC   XYDFC    ZDFC

ON      ON      ON

Horizontal Diffusion (HDIFF)

Field		Name		Value						Description

1			XYDF		Real			Value for horizontal diffusion (m2 s-1)

2			ZDFMUL	Real			Multiplier for vertical diffusion

3			ZDFMAX	Real			Maximum vertical diffusion (m2 s-1)

The first application of the water-quality model was in a system with negligible horizontal dispersion.  To minimize the size of the hydrodynamic file, dispersion was specified once in the control file rather than written out by the hydrodynamic model at every cell interface, at every time step.  This feature is still in place when the BINARY or DEPTH_AV hydrodynamic options are employed.   Spatially-variable dispersion is a feature of the model code but the user must perform revisions to read dispersion in binary format.  When the ASCII option is employed, spatially- and temporally-varying horizontal dispersion is specified in the hydrodynamic file and the field on this card is ignored.

In the first application of the model, sensitivity runs were performed with varying vertical diffusion.  To avoid running the hydrodynamic model repeatedly, a vertical diffusion multiplier was installed in the water-quality model.  For similar reasons, specification of maximum vertical diffusion was allowed.

EXAMPLE

HDIFF       XYDF  ZDFMUL  ZDFMAX

2.0    1.00     0.1

Constituent Input (CST INPUT)

Field		Name		Value					Description

1			BCC		Character		Turns on/off boundary conditions

2			S1C		Character		Turns on/off first external load file

3			S2C		Character		Turns on/off second external load file

4			S3C		Character		Turns on/off third external load file

5			MDC		Character		Turns on/off modifications to initial 											concentrations

6			BFC		Character		Turns on/off specified benthic fluxes

7			ATMC		Character		Turns on/off atmospheric inputs

8			SAVLC		Character		Turns on/off submerged aquatic 											plant inputs

This card allows the user to selectively turn on/off constituent inputs.  All fields must be specified as " ON" or "OFF".

# EXAMPLE

CST INPUT    BCC     S1C     S2C     S3C     MDC     BFC    ATMC   SAVLC

ON      ON      ON     OFF     OFF     OFF      ON     OFF

Nutrient Reductions (NUTR RED)

Field		Name		Value						Description

1			REDS1C	Real			Modifies S1 carbon inputs

2			REDS1N	Real			Modifies S1 nitrogen inputs

3			REDS1P	Real			Modifies S1 phosphorus inputs

4			REDS2C	Real			Modifies S2 carbon inputs

5			REDS2N	Real			Modifies S2 nitrogen inputs

6			REDS2P	Real			Modifies S2 phosphorus inputs

7			REDS3C	Real			Modifies S3 carbon inputs

8			REDS3N	Real			Modifies S3 nitrogen inputs

9			REDS3P	Real			Modifies S3 phosphorus inputs

This card allows the user to selectively modify nutrient loads.  Values in the input files are multiplied by the fraction specified on this card.  Specification of zero eliminates input values.  Specification of unity leaves input values as in the original files.  This feature is useful in preliminary investigations of nutrient reduction strategies.

EXAMPLE

LOAD RED  REDS1C  REDS1N  REDS1P  REDS2C  REDS2N  REDS2P  REDS3C  REDS3N  REDS3P

1.0     1.0     1.0     1.0     1.0     1.0     1.0     1.0     1.0

Boundary Condition Reductions (BNDC RED)

Field		Name		Value						Description

1			REDCBC	Real			Modifies boundary carbon inputs

2			REDCBN	Real			Modifies boundary nitrogen inputs

3			REDCBP	Real			Modifies boundary phosphorus inputs

This card allows the user to selectively modify boundary conditions.  Values in the input files are multiplied by the fraction specified on this card.  Specification of zero eliminates input values.  Specification of unity leaves input values as in the original files.  This feature is useful in preliminary investigations of nutrient reduction strategies.

# EXAMPLE

BNDC RED  REDCBC  REDCBN  REDCBP

1.0     1.0     1.0

Boundary Concentrations (BOUNDARY)

Field		Name		Value					Description

1			BNDCC	Character			Specifies how boundary

concentrations are handled

This card specifies if input boundary concentrations are updated when the next value is read in from the time-varying data file or if the concentrations are interpolated for each timestep.  The inputs are "STEP" or "INTERP".  Information regarding these options is found in Chapter 2.

EXAMPLE

BOUNDARY   BNDCC

INTERP

Active Constituents (ACT CST)

Field		Name		Value				Description

1-27		ACC		Character		Turns on/off constituent calculations

This card allows the user to turn on/off a state variable for a simulation.  Care must be taken when exercising this option because of the interaction of the state variables in the kinetic formulations.  Most commonly, this option is used when initially calibrating the water-quality model transport to the hydrodynamic model transport using salinity.  This option turns off computations only.  All state variables must still be included in load files, boundary conditions, etc.  The state variables and their order are

1		Temperature

2		Salinity

3		Suspended Solids

4		Algal Group One

5		Algal Group Two

6		Algal Group Three

7		Zooplankton One

8		Zooplankton Two

9		Dissolved organic carbon

10		Labile particulate carbon

11		Refractory particulate carbon

12		Ammonium

13		Nitrate-nitrite

14		Dissolved organic nitrogen

15		Labile particulate nitrogen

16		Refractory particulate nitrogen

17		Total phosphate

18		Dissolved organic phosphorus

19		Labile particulate phosphorus

20		Refractory particulate phosphorus

21		Chemical oxygen demand

22		Dissolved oxygen

23		Particulate silica

24		Dissolved silica

25		Pathogen

26		Toxic One

27		Toxic Two

EXAMPLE

ACT CST      ACC     ACC     ACC     ACC     ACC     ACC     ACC     ACC     ACC

ON      ON      ON     OFF     OFF      ON     OFF     OFF      ON

ON      ON      ON      ON      ON      ON      ON      ON      ON

ON      ON      ON      ON     OFF     OFF      ON     OFF     OFF

Number of Files (# FILES)

Field		Name		Value						Description

1			NHYDF	Integer				Number of hydrodynamic files in

simulation

2			NTVDF	Integer			      Number of time-varying data

files in simulation

This card is used to specify the number of time-varying input files, which will be used during the simulation.  When the end of a file is reached during the simulation, a new file will be opened.  For example, if NTVDF is set to 3, then a new file will be opened each time an end of file is reached until the third file is opened.  If an end of file is reached for the last file, the run terminates.  Potential time-varying input files are HYD through BFI listed on the next page.

EXAMPLE

# FILES    NHYDF   NTVDF

7       7

Input/Output Filenames

Field		Name		Value						Description

1			"File"		Character				Input/output filename

The last cards in the control file specify the names of all input/output files used by the model.  This method eliminates the need to know how to link FORTRAN files under a particular operating system.  However, it does not preclude linking I/O files.  If the user does decide to link files, they must be linked with the names specified by the user on the following cards.  Under UNIX and Windows operating systems, full pathnames can be included in the filename specification.   The number of hydrodynamic files listed under HYDFN must correspond to the number specified for NHYDF.  The same holds true for the other time-varying input files.  There must be the exact number specified by NTVDF.

Input File Names

MAPFN		Map file associating model cells with appropriate flow faces.

GEOFN		Geometry file associating cells with their vertical neighbors and

associating surface and bottom cells.

ICIFN		Initial conditions file.

AGRFN		Algal growth rate file.

ZOOFN		Zooplankton input file.

SUSFN		Suspension feeders input file.

STLFN		Settling rate file.

MRLFN		Mineralization rate file.

PPTFN		Pathogen or Toxics input file.

EXTFN		Light Extinction input file.

HYDFN		Hydrodynamic input file.

METFN		Meteorological input file.

S1FN		Source One input file.

S2FN		Source Two input file.

S3FN		Source Three input file.

ATMFN		Atmospheric input file.

SAVFN		Submerged Aquatic Vegetation input file.

CBCFN		Boundary Conditions input file.

BFIFN		Benthic Flux input file.

#### Output File Names

ICOFN		Initial Conditions output file.

SNPFN		ASCII constituent output file.

RSOFN		Restart output file.

PLTFN		Instantaneous Binary constituent output file.

APLFN		Averaged Binary constituent output file.

DIAFN		Diagnostics output file.

TFLFN		Transport Flux output file.

KFLFN		Kinetic Flux output file.

OPLFN		Dissolved Oxygen Volume Days output file.

MBLFN		Mass Balance output file.

ALOFN		Algal output file.

ZFOFN		Zooplankton output file.

BFOFN		Benthic Flux output file.

SVOFN		Submerged Aquatic Vegetation output file.

SUDFN		Suspension Diagnostics output file.

PTOFN		Pathogens and Toxics output file.

Input/Output Filenames

(Continued, Non-Time Varying Input files)

# EXAMPLE

MAP FILE................................MAPFN...................................

../real_inputs/map_LW_11_01_01.inp

GEO FILE................................GEOFN...................................

../real_inputs/geo_LW_11_01_01.inp

ICI FILE................................ICIFN...................................

inputs/wqm_ico.jul23

AGR FILE................................AGRFN...................................

../real_inputs/wqm_agr_LW.jul18

ZOO FILE................................ZOOFN...................................

wqm_zoo.none

SUS FILE................................SUSFN...................................

wqm_sfi.none

STL FILE................................STLFN...................................

../real_inputs/wqm_stl_LW.jul23

MRL FILE................................MRLFN...................................

../inputs/wqm_mrl_LW.jul18

PTT FILE................................PTTFN...................................

../real_inputs/wqm_ptt.june21

EXT FILE................................EXTFN...................................

../real_inputs/wqm_kei_LW.oct22

Input/Output Filenames

(Continued, Time Varying Hydrodynamic files)

# EXAMPLE

HYD FILE................................HYDFN...................................

/cc2/kim/lake_washington/1995/HYD.INP_0515

/cc2/kim/lake_washington/1996/HYD.INP_0515

/cc2/kim/lake_washington/1997/HYD.INP_0515

/cc2/kim/lake_washington/1995/HYD.INP_0515

/cc2/kim/lake_washington/1996/HYD.INP_0515

/cc2/kim/lake_washington/1997/HYD.INP_0515

/cc2/kim/lake_washington/1995/HYD.INP_0515

Input/Output Filenames

(Continued, Time Varying Input files)

MET FILE................................METFN...................................

../real_inputs/wqm_met_LW.95

../real_inputs/wqm_met_LW.96

../real_inputs/wqm_met_LW.97

../real_inputs/wqm_met_LW.95

../real_inputs/wqm_met_LW.96

../real_inputs/wqm_met_LW.97

../real_inputs/wqm_met_LW.95

S1  FILE................................S1FN....................................

../real_inputs/wqm_nps_LW.sept16

../real_inputs/wqm_nps_LW.sept16

../real_inputs/wqm_nps_LW.sept16

../real_inputs/wqm_nps_LW.sept16

../real_inputs/wqm_nps_LW.9597

../real_inputs/wqm_nps_LW.9597

../real_inputs/wqm_nps_LW.9597

S2  FILE................................S2FN....................................

../real_inputs/wqm_CSO_LW.9597

../real_inputs/wqm_CSO_LW.9597

../real_inputs/wqm_CSO_LW.9597

../real_inputs/wqm_CSO_LW.9597

../real_inputs/wqm_CSO_LW.9597

../real_inputs/wqm_CSO_LW.9597

../real_inputs/wqm_CSO_LW.9597

S3  FILE................................S3FN....................................

wqm_atm.none

wqm_atm.none5

wqm_atm.none

wqm_atm.none

wqm_atm.none5

wqm_atm.none

wqm_atm.none5

ATM FILE................................ATMFN...................................

../real_inputs/wqm_atm.LW

../real_inputs/wqm_atm.LW

../real_inputs/wqm_atm.LW

../real_inputs/wqm_atm.LW

../real_inputs/wqm_atm.LW

../real_inputs/wqm_atm.LW

../real_inputs/wqm_atm.LW

SAV FILE................................SAVFN...................................

wqm_sav.none

wqm_sav.none

wqm_sav.none

wqm_sav.none

wqm_sav.none

wqm_sav.none

wqm_sav.none

CBC FILE................................CBCFN...................................

../real_inputs/wqm_cbc_LW.sept16

../real_inputs/wqm_cbc_LW.sept16

../real_inputs/wqm_cbc_LW.sept16

../real_inputs/wqm_cbc_LW.sept16

../real_inputs/wqm_cbc_LW.sept16

../real_inputs/wqm_cbc_LW.sept16

../real_inputs/wqm_cbc_LW.sept16

BFI FILE................................BFIFN...................................

../real_inputs/wqm_bfi_LW.jul23

../real_inputs/wqm_bfi_LW.jul23

real_inputs/wqm_bfi_LW.ini

wqm_bfi.none

real_inputs/wqm_bfi_LW.ini

wqm_bfi.none

wqm_bfi.none

Input/Output Filenames

(Continued, Output files)

EXAMPLE

ICO FILE................................ICOFN...................................

outputs/wqm_ico.jul23

SNP FILE................................SNPFN...................................

wqm_snp.LW_opt

RSO FILE................................RSOFN...................................

outputs/wqm_rso.LW_opt

PLT FILE................................PLTFN...................................

outputs/wqm_plt.LW_opt

APL FILE................................APLFN...................................

outputs/wqm_apl.jul23

DIA FILE................................DIAFN...................................

outputs/wqm_dia.jul23

TFL FILE................................TFLFN...................................

outputs/wqm_tfl.jul23

KFL FILE................................KFLFN...................................

outputs/wqm_kfl.jul23

OPL FILE................................OPLFN...................................

outputs/wqm_opl.LW_opt

MBL FILE................................MBLFN...................................

outputs/wqm_mbl.LW_opt

ALO FILE................................ALOFN...................................

outputs/wqm_alo.LW_opt

ZFO FILE................................ZFOFN...................................

outputs/wqm_zfo.LW_opt

BFO FILE................................BFOFN...................................

outputs/wqm_bfo.LW_opt

SVO FILE................................SVOFN...................................

outputs/wqm_svo.LW_opt

SUD FILE................................SUDFN...................................

outputs/wqm_sfo.LW_opt

PTO FILE................................PTOFN...................................

outputs/wqm_pto.LW_opt