## External Loads Files

Three files are available for the input of external loads.  The format of the files is identical.  Provision of three files allows separation of loads by source e.g. point-source and nonpoint-source.  Three files also allow updates of loads at different time intervals.  One file can list loads at daily intervals while the others lists loads at monthly intervals.

The first two lines in each External Loads file are reserved for identification.  The remainder of each file lists the number of loads for each constituent, the cell location for each load, and the loading rate for each cell.  Multiple inputs into single cells are allowed.  Once specified, loads are held constant until updated.  Loads may be updated at arbitrary intervals.  The total number of loads and their location cannot be altered.

Number of Loads (NPSLN)

Field		Name		Value			Description

1-27		NPSLN		Integer		Number of external loads for

each constituent

This input is required only once at the beginning of the file.  The number of loads must be specified for each of 27 state variables, even though not all are active.  The format is (8X,9I8).  A blank line and a header line precede this input.

EXAMPLE

NPSLN   NPSLN   NPSLN   NPSLN   NPSLN   NPSLN   NPSLN   NPSLN   NPSLN

0       0      41       0       0       0       0       0      41

41      41      41      41      41      41      41      41      41

41      41       0       0       0       0      41       0       0

Location of Loads (NPSLB)

Field		Name		Value				Description

1-9			NPSLB		Integer		  Cell receiving external load

At least one specification is required for each of the 27 state variables, even though not all are active.  The number of cells receiving loads should agree with the variable NPSLN.  The format is (8X,9I8).  A blank line and a header line precede the specification for each variable.  The first 8 columns are not read in and may be used as a code to identify state variables.  Variables are read in the order of active constituents:

1		Temperature

2		Salinity

3		Suspended Solids

4		Algal Group One

5		Algal Group Two

6		Algal Group Three

7		Zooplankton One

8		Zooplankton Two

9		Dissolved organic carbon

10		Labile particulate carbon

11		Refractory particulate carbon

12		Ammonium

13		Nitrate-nitrite

14		Dissolved organic nitrogen

15		Labile particulate nitrogen

16		Refractory particulate nitrogen

17		Total phosphate

18		Dissolved organic phosphorus

19		Labile particulate phosphorus

20		Refractory particulate phosphorus

21		Chemical oxygen demand

22		Dissolved oxygen

23		Particulate silica

24		Dissolved silica

25		Pathogen

26		Toxic One

27		Toxic Two

EXAMPLE

ISS   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

620      86     533     628    1256     629     599     588     549

86     126     104     216     176      29     135      75     232

877    1493    2095    2669    3224      19       4      27     681

189     297     610    1240    1851    2444    3012    3564       2

657    1285    1890    2480    3048

External Load (NPSL)

A blank line and a header line begin the specification of external loads.  Loads can be updated at arbitrary intervals and must be specified at least twice.  The first time is at day zero.  If the user wishes to maintain the initial loads throughout the run, the second specification should be on a Julian day greater than the duration of the model run.  If the final specification is on a Julian day less than the run duration, the model code will attempt to open a succeeding external loads file when the last Julian day is exceeded.

The first line of loads for each variable includes the Julian day on which the load applies.  This day should be the same for each grouping of 27 variables.  For convenience, a brief identification code can be placed in the first 8 columns.  This field is skipped by the model READ statement, however (8X,9F8.0,:/(:16X,8F8.0)).

Field	Name		Value		Description

1	Code		Character	A variable abbreviation or code (skipped

by FORMAT statement)

2	JDAY		Real		Julian day for update of loads

3 – 10	KG/DAY	Real		Load (kg d-1, except pathogen in 106

organisms d-1)

# Example

NPS LOAD    JDAY  KG/DAY  KG/DAY  KG/DAY  KG/DAY  KG/DAY  KG/DAY  KG/DAY  KG/DAY

ISS      0.    155.     11.     21.     17.     17.      5.     19.     32.

45.     30.     60.     37.     23.      5.     40.      2.

7.      2.      2.      2.      2.      2.      2.    325.

6.     51.     51.    236.  24996.    137.    137.    137.

137.    137.    137.  10289.  10289.  10289.  10289.  10289.

10289.

The following pages illustrate a complete external load file.  Loads are initially specified at Julian day 0 and updated at day 30.

Point-source example input file

Chesapeake Bay Workshop

# LOADS    NPSLN   NPSLN   NPSLN   NPSLN   NPSLN   NPSLN   NPSLN   NPSLN   NPSLN

0       0       0       5       0       0       0       0       5

5       5       5       5       5       5       5       5       5

5       5       0       0       0       0       0       0       0

TEMP   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

0

SALT   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

0

ISS   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

0

CYAN   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

1       2       3       4       5

DIAT   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

0

GREN   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

0

ZOO1   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

0

ZOO2   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

0

DOC   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

1       2       3       4       5

LPOC   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

1       2       3       4       5

RPOC   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

1       2       3       4       5

NH4   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

1       2       3       4       5

NO3   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

1       2       3       4       5

DON   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

1       2       3       4       5

LPON   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

1       2       3       4       5

RPON   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

1       2       3       4       5

PO4   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

1       2       3       4       5

DOP   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

1       2       3       4       5

LPOP   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

1       2       3       4       5

RPOP   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

1       2       3       4       5

COD   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

0

DO   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

0

PSIL   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

0

DSIL   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

0

PATH   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

0

TOX1   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

0

TOX2   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB   NPSLB

0

NPS LOAD   JDAY     PSL     PSL     PSL     PSL     PSL     PSL     PSL     PSL

TEMP      0.      0.

SALT      0.      0.

ISS      0.      0.

CYAN      0.      0.

DIAT      0.      0.

GREEN      0.      0.

ZOO1      0.      0.

ZOO2      0.      0.

DOC      0.     10.     10.     10.     10.     10.

LPOC      0.     18.     10.     10.     10.     10.

RPOC      0.     10.     10.     10.     10.     10.

NH4      0.     10.     10.     10.     10.     10.

NO3      0.     10.     10.     10.     10.     10.

DON      0.     10.     10.     10.     10.     10.

LPON      0.     10.     10.     10.     10.     10.

RPON      0.     10.     10.     10.     10.     10.

PO4      0.     10.     10.     10.     10.     10.

DOP      0.     10.     10.     10.     10.     10.

LPOP      0.     10.     10.     10.     10.     10.

RPOP      0.     10.     10.     10.     10.     10.

COD      0.      0.

DO      0.      0.

PSIL      0.      0.

DSIL      0.      0.

PATH      0.      0.

TOX1      0.      0.

TOX2      0.      0.

TEMP     30.      0.

SALT     30.      0.

ISS     30.      0.

CYAN     30.      0.

DIAT     30.      0.

GREEN     30.      0.

ZOO1     30.      0.

ZOO2     30.      0.

DOC     30.     20.     20.     20.     20.     20.

LPOC     30.     18.     20.     20.     20.     20.

RPOC     30.     20.     20.     20.     20.     20.

NH4     30.     20.     20.     20.     20.     20.

NO3     30.     20.     20.     20.     20.     20.

DON     30.     20.     20.     20.     20.     20.

LPON     30.     20.     20.     20.     20.     20.

RPON     30.     20.     20.     20.     20.     20.

PO4     30.     20.     20.     20.     20.     20.

DOP     30.     20.     20.     20.     20.     20.

LPOP     30.     20.     20.     20.     20.     20.

RPOP     30.     20.     20.     20.     20.     20.

COD     30.      0.

DO     30.      0.

PSIL     30.      0.

DSIL     30.      0.

PATH     30.      0.

TOX1     30.      0.

TOX2     30.      0.